TASK-DTI-00

CAPABILITY & CAPTURE BASELINE REPORT

CaptivPortal — Device Traffic Enrichment / Device Intelligence

Research / Lab / Inventory
PRE-REMEDIATION SNAPSHOT

Evidence date: 2026-10-05 (AZST / UTC+4)
Audience: Owner, Architect, Tech Lead
Report status: complete research handoff for review

# Document Control

| Field | Value |
| --- | --- |
| Project | CaptivPortal |
| Program | Device Traffic Enrichment / Device Intelligence (DTI) |
| Task | TASK-DTI-00 — Capability & Capture Baseline |
| Mode | RESEARCH / LAB / INVENTORY |
| Implementation authorization | NO |
| Production feature activation | NO |
| Primary live evidence date | 2026-10-05 |
| Status | FOR ARCHITECT + TECH LEAD REVIEW — pre-remediation |
| Primary contract | TASK-DTI-00-CAPABILITY-CAPTURE-BASELINE-SYSADMIN-FINAL.md |
| Architecture reference | DEVICE-TRAFFIC-ENRICHMENT-ARCHITECTURE-v0.3_FINAL_TECHLEAD_ACCEPTED.md |

Canonical evidence numbering: This report canonicalizes Evidence IDs E01–E41. Interim chat labels were working labels and are not authoritative.

# 1. Executive Summary

TASK-DTI-00 successfully established the actual current production mirror/capture path, its placement, software consumers, resource characteristics, limitations, and several production defects that were not assumptions in the original plan. The research did not implement DTI and did not alter SPAN/RSPAN, VLAN, routing, NAT, Fingerprint or Suricata production configuration. The final research verdict is PARTIAL, not because the capture path is unknown, but because a controlled owner-client trace was intentionally deferred, RSPAN duplication remains unrepaired at evidence freeze, and two architecture-admission decisions (Suricata coexistence and authoritative periodic traffic accounting) cannot be honestly marked proven without a separate post-research change/validation step.

| Area | Result |
| --- | --- |
| Capture topology | PROVEN end-to-end: Catalyst RSPAN → omv-hp → VLAN999/macvtap → Ubuntu VM enp8s0. |
| Placement | PROVEN pre-Cisco-NAT; client MAC/private IPv4 and both traffic directions are visible. |
| RSPAN defect | CONFIRMED packet duplication already present at physical RSPAN ingress, before macvtap/VM. |
| Fingerprint protection | PROVEN exact-frame dedup (0.25 ms, 65,536-entry bounded cache) before raw DHCP/TCP-SYN normalization. |
| Live duplicate impact | ~52.29% cumulative duplicate ratio in the Fingerprint raw filtered candidate stream; MUST NOT be extrapolated to all VLAN999 traffic. |
| Capture quality | Current 60 s sample clean: no new CRC/no-buffer/missed/fifo/VLAN999 drops; historical receive loss exists. |
| Current mirror load | ~156.6 Mbit/s VLAN999 average during measured 60 s; physical NIC total RX ~255.9 Mbit/s. |
| Suricata | 8.0.6; production emits TLS/QUIC/stats; installed version supports flow/DNS/DHCP/HTTP and other EVE families. |
| Resource headroom | Current Fingerprint+Suricata ~0.96 CPU-seconds/60 s (~1.6% of one core) and ~74 MiB RAM; VM has substantial RAM/CPU headroom with moderate I/O wait. |
| Storage | Fingerprint spool physical ~89.4 MiB but ~89.34 MiB is SQLite freelist; active queue footprint negligible at sample time. |
| Site binding | PROVEN: zefer-span-01 → site 6a64f17630da7c70d232187a → guest 192.168.8.0/22. |
| NetFlow | Diagnostic only. Cisco exports to .222:2055 and .202:9995; .202:9995 packets arrive but are dropped by host nftables before nfcapd. |
| Zeek | NO baseline justification; no concrete gap established. |
| Mandatory follow-up | After report/review: repair RSPAN duplication, then revalidate both directions, VLAN20 visibility, Fingerprint, Suricata, drops and accounting candidate semantics. |

Important decision refinement: The interim hypothesis “Suricata-only traffic accounting” is not treated as a final proven decision. The accepted architecture explicitly warns that terminal flow events may be insufficient for current rate, current activity, visit-boundary slicing and long-lived-flow accounting. Final DTI-00 position: Suricata is the preferred general metadata engine; authoritative periodic traffic accounting source remains UNRESOLVED pending the next approved validation.

# 2. Scope, Method and Safety Boundary

The research object was the currently operational production SPAN/RSPAN / Device Fingerprint capture infrastructure. Future server designs were explicitly excluded as current facts. Evidence was classified as OBSERVED, CONFIGURATION-PROVEN, CONTROLLED-TRACE-PROVEN, MODELED, UNKNOWN, or NOT_APPLICABLE.

Preferred mode: read-only inspection + bounded passive observation.

No production SPAN/RSPAN, VLAN, trunk, routing, NAT, Omada, Fingerprint, Suricata or firewall configuration was changed.

No persistent full PCAP or arbitrary visitor payload was collected.

One explicit diagnostic exception was approved by Owner: install tcpdump on omv-hp for bounded evidence collection.

Owner-controlled client trace was intentionally deferred after the existing evidence had already proved MAC/IP visibility, bidirectionality and pre-NAT placement. Re-open only if a later decision requires identity-correlated proof not available from existing evidence.

# 3. Current Production Capture Topology

Guest client / Zefer_Parki
        | VLAN20 (192.168.8.0/22)
        v
EAP613 AP (observed AP attachment on Catalyst Gi1/0/10)
        | 802.1Q trunk; native VLAN10; allowed 10,20,50
        v
Cisco2960 WS-C2960S-48LPS-L
        |
        +--> ORIGINAL TRAFFIC: Gi1/0/1 trunk --> Cisco3845 Gi0/1.20
        |       gateway 192.168.10.1/22, MAC 00:24:97:7b:b9:41
        |       ip nat inside
        |       --> Gi0/0 192.168.100.10 (ip nat outside) --> upstream
        |
        +--> RSPAN session 1
                source VLANs: 10,20  direction: BOTH
                destination remote VLAN: 999 (RSPAN_DEVICE_FP)
                |
                v
             Gi1/0/48 trunk (native 40; allowed 40,999; 1Gb/full)
                |
                v
             omv-hp / enp7s0f1 (Intel 82576 / HP NC362i)
                | vlan999 tagged on physical capture API
                v
             enp7s0f1.999 --> macvtap0 (passthru)
                |
                v
             VM "Ubunt" / local.domain.az
                |
                +--> enp1s0 = 192.168.0.202 management/application
                |
                +--> enp8s0 = capture NIC, no IP
                       |
                       +--> fingerprint-sensor AF_PACKET + frozen BPF + exact dedup
                       |      DHCP + guest outbound TCP SYN
                       |
                       +--> Suricata 8.0.6 AF_PACKET workers
                              --> EVE unix_dgram socket
                              --> fingerprint-sensor TLS/QUIC/stats normalization

## 3.1 Switch and RSPAN facts

| Item | Observed/configured value |
| --- | --- |
| Catalyst | Cisco2960; WS-C2960S-48LPS-L; IOS 15.2(2)E9; serial FOC1846Z2UF |
| RSPAN session | Session 1, Remote Source Session |
| Source scope | VLAN10 + VLAN20, Both directions |
| RSPAN VLAN | 999 / RSPAN_DEVICE_FP / Remote SPAN VLAN Enabled |
| Destination transport | Gi1/0/48; 802.1Q trunk; native VLAN40; allowed/active/forwarding VLAN40,999; 1Gb/full |
| Guest AP port | Gi1/0/10, description AP - EAP613 NEW; trunk native VLAN10; allowed 10,20,50 |
| Gateway-facing port | Gi1/0/1 trunk; allowed 10,20,30,50 |
| Router gateway MAC | 0024.977b.b941 learned on Gi1/0/1 for VLAN10/20/30 |

# 4. HostCapturePathProof

The physical switch destination and the VM capture interface were independently correlated by MAC addresses and libvirt topology. This proves the complete host-side delivery path rather than assuming a historic interface name.

| Layer | Evidence |
| --- | --- |
| Catalyst Gi1/0/48 | VLAN40 MAC 44:1e:a1:61:e8:d9 learned on port |
| omv-hp physical NIC | enp7s0f1 MAC 44:1e:a1:61:e8:d9 — exact match |
| RSPAN subinterface | enp7s0f1.999, VLAN protocol 802.1Q id 999, UP |
| macvtap | macvtap0@enp7s0f1.999, passthru, MAC 52:54:00:59:23:b8 |
| VM libvirt mapping | Ubunt direct macvtap0 / enp7s0f1.999, virtio MAC 52:54:00:59:23:b8 |
| VM capture NIC | enp8s0 MAC 52:54:00:59:23:b8, UP, no IP |
| VM management NIC | enp1s0 MAC 52:54:00:0d:2f:54, 192.168.0.202/24 |

# 5. Capture Placement, Identity and Directionality

Guest client private IPv4 and client Ethernet MAC are visible at enp8s0.

Both client→remote and remote→client traffic were passively observed.

Original guest VLAN tag is not visible at the VM capture API; the RSPAN transport VLAN999 tag is visible on the physical host NIC and is removed by the Linux .999 subinterface as expected.

Capture is conclusively before Cisco3845 NAT. The router guest subinterface is Gi0/1.20 = 192.168.10.1/22 with ip nat inside; Gi0/0 = 192.168.100.10/24 with ip nat outside.

The upstream public-NAT location beyond Cisco3845 was not required and was not claimed.

# 6. Mirror Duplication Analysis

Major finding: Exact packet/frame duplication is CONFIRMED and reproducible. It is visible on the VM capture interface and already present on omv-hp physical RSPAN ingress, therefore macvtap and the VM are not the source of the initial duplication.

Observed evidence included repeated frames with effectively identical timing and packet identity. Some equivalent L3 packets were also seen with different Ethernet MAC pairs, consistent with the same routed traffic being observed at more than one mirrored L2 context. The strongest root-cause candidate is the combination of source VLAN10 + VLAN20 with direction BOTH in a router-on-a-stick topology. This root cause is strongly supported but was not finalized by reconfiguring production RSPAN, because that is outside DTI-00.

| Aspect | Finding |
| --- | --- |
| Duplication at VM enp8s0 | CONFIRMED |
| Duplication at omv-hp enp7s0f1 VLAN999 | CONFIRMED |
| VM/macvtap as initial source | EXCLUDED by physical-host evidence |
| Likely source domain | Catalyst RSPAN source semantics / monitored topology |
| Exact production fix | DEFERRED until after report + Architect/Tech Lead review |
| Impact | Distorted packet/byte/event accounting; unnecessary parser load; misleading DTI measurements if uncorrected |

# 7. Existing Fingerprint Sensor: Raw Path, Dedup and Suricata Path

## 7.1 Two parallel packet consumers

Linux /proc/net/packet and process FD mapping proved two independent direct packet consumers on enp8s0: fingerprint-sensor and Suricata. Suricata additionally feeds EVE events back to fingerprint-sensor over a Unix datagram socket.

enp8s0
  +--> fingerprint-sensor raw AF_PACKET socket
  |      --> frozen BPF
  |      --> exact-frame dedup
  |      --> DHCP / TCP-SYN normalization
  |
  +--> Suricata AF_PACKET workers
         --> EVE unix_dgram
         --> fingerprint-sensor TLS / QUIC / stats normalization

## 7.2 Frozen raw-capture filter

(udp and (port 67 or port 68)) or
(src net 192.168.8.0/22 and tcp[tcpflags] & (tcp-syn|tcp-ack) == tcp-syn)

Therefore Fingerprint raw_packet_count is NOT a whole-mirror traffic counter. It covers DHCP and guest outbound TCP SYN candidates only.

## 7.3 Exact-frame deduplication

| Parameter | Value |
| --- | --- |
| Algorithm | BLAKE2s over capture_source_id + frame length + exact frame bytes |
| Digest size | 16 bytes |
| Default horizon | 0.25 ms |
| Maximum cache entries | 65,536 |
| Placement | Before DHCP/TCP-SYN normalization |
| Behavior | Duplicate candidate is dropped from raw normalization path |

Live production telemetry showed raw_dedup_ratio ≈ 0.5229. Example cumulative point at 11:49:19 AZST: raw_packet_count=135,955; duplicate_packet_count=71,091; deduplicated_packet_count=64,864. This proves strong duplicate impact in the FILTERED Fingerprint candidate stream only; it MUST NOT be extrapolated to all VLAN999 packets.

# 8. Suricata Inventory and Capability

| Item | Result |
| --- | --- |
| Version | Suricata 8.0.6 RELEASE |
| Capture mode | AF_PACKET on enp8s0, workers |
| Current EVE transport | unix_dgram → /run/captive-portal/fingerprint-sensor/suricata-eve.sock |
| Current enabled EVE types | TLS (JA4, client_alpns, client_handshake), QUIC, stats |
| Installed-version capabilities observed | flow, DNS, DHCP, HTTP, TLS, QUIC, stats, alert; AF_PACKET; JA4; Hyperscan; Unix socket |
| Not enabled in current Fingerprint config | flow, DNS, HTTP, alert |
| Current Fingerprint consumption | TLS/QUIC normalized to evidence; stats used as Suricata heartbeat/capture health |

Coexistence boundary: The accepted Fingerprint Suricata configuration is protected. DTI-00 did not enable flow/DNS/HTTP or otherwise mutate fingerprint-suricata.service.

# 9. NIC / Kernel Presentation and Capture Quality

| Item | Value |
| --- | --- |
| NIC | Intel 82576 Gigabit Network Connection / HP NC362i integrated dual-port adapter |
| Interface | omv-hp enp7s0f1; PCI 0000:07:00.1 |
| Driver / firmware | igb / firmware 1.8.2 |
| Link | 1000 Mb/s, Full Duplex, autoneg on |
| RX offloads/presentation | rx-checksum ON; GRO ON; LRO OFF; rx-vlan-offload ON; RX hash ON |
| TX/GSO | TSO ON; GSO ON; scatter-gather ON |
| RX ring | current 256; maximum 4096 |
| RSPAN physical presentation | 802.1Q VLAN999 observed at enp7s0f1; untagged at enp7s0f1.999 / VM as expected |

## 9.1 Historical counters

| Counter | Observed cumulative value / interpretation |
| --- | --- |
| rx_crc_errors | 1 |
| rx_no_buffer_count | 2,074 |
| rx_missed_errors | 46,121 |
| rx_fifo_errors | 46,121 |
| per-RX-queue drops | 0 on all observed queues |
| Interpretation | Historical receive loss exists; not proof of current sustained loss |

## 9.2 60-second natural-load interval

| Metric | Physical enp7s0f1 | VLAN999 |
| --- | --- | --- |
| Packets delta | 1,829,066 | 1,151,004 |
| Average packets/s | ~30,484 | ~19,183 |
| Bytes delta | 1,918,930,440 | 1,174,563,011 |
| Average rate | ~255.9 Mbit/s | ~156.6 Mbit/s |
| New rx_missed/fifo/no-buffer/CRC | 0 | — |
| New rx_dropped | — | 0 |

The current sample was clean at substantial natural load. This does not prove lifetime completeness. The physical interface also carries non-RSPAN traffic (native VLAN40), so physical total RX is not equal to VLAN999 traffic.

# 10. Mirror Capacity / Headroom

The destination link is 1 Gb/s full-duplex. VLAN999 averaged ~156.6 Mb/s in the bounded 60 s sample; physical RX averaged ~255.9 Mb/s. This demonstrates current headroom in the sampled window. However source VLAN10+20/BOTH can in principle aggregate above a single 1 Gb/s destination, and no source-side counter reconciliation was performed over the same window. Therefore oversubscription is POSSIBLE, not proven absent.

Required semantic: CAPTURE_QUALITY_BASELINE=PASS means monitoring/evidence quality is sufficient for the baseline. It does NOT mean 100% packet completeness.

# 11. Cisco3845 / First L3 / NAT

| Item | Observed value |
| --- | --- |
| Router | Cisco3845; IOS 15.1(4)M12a; serial FOC130427S0 |
| Guest first L3 | Gi0/1.20, dot1Q 20, 192.168.10.1/22, MAC 00:24:97:7b:b9:41 |
| Guest ACL | GUEST_TTL_FILTER inbound |
| Flow accounting | ip flow ingress + ip flow egress |
| NAT inside | Gi0/1.10, Gi0/1.20, Gi0/1.30 |
| NAT outside | Gi0/0, 192.168.100.10/24 |
| NAT rule | ip nat inside source list 100 interface Gi0/0 overload |
| Capture relationship | RSPAN sees guest traffic before Cisco NAT |

# 12. NetFlow / nfdump Overlap and Additional Defect

NetFlow was investigated as diagnostic/cross-check evidence, not as the primary device identity source.

| Finding | Evidence / status |
| --- | --- |
| Cisco exporter version | NetFlow v9 |
| Actual current destinations | 192.168.0.222:2055 and 192.168.0.202:9995 |
| Difference from earlier stated expectation | Earlier scope mentioned 192.168.0.220:2055; live router proves .222:2055 + .202:9995 |
| .202:9995 listener | nfcapd PID/service present; /var/nfdump configured; historical directory ~16G |
| .202:9995 current ingest | 0 flows; current nfcapd files ~276 B/header-only |
| Network delivery to .202:9995 | PROVEN: UDP datagrams from 192.168.0.1 reach enp1s0 |
| Root cause of zero ingest | CONFIGURATION-PROVEN host firewall: nftables INPUT policy DROP; no UDP/9995 accept rule |
| .202 second local nfcapd | UDP/2055 via nfdump@default.service → /var/cache/nfdump (~88M historical); not current Cisco destination |
| .222:2055 | Owner confirmed PRTG; further inspection intentionally excluded as unrelated to DTI primary path |
| DTI role | DIAGNOSTIC / cross-check only |

Post-review option: If .202:9995 NetFlow is retained as a DTI diagnostic input, fix its firewall admission separately. Do not fix merely because the defect exists; first confirm the path is still required.

# 13. CaptureScopeBinding / Site Mapping

| Binding element | Proven value |
| --- | --- |
| capture_source_id | zefer-span-01 |
| sensor site_id | 6a64f17630da7c70d232187a |
| guest CIDR | 192.168.8.0/22 |
| Same Site ID in CaptivPortal | CAPPORT config, Fingerprint sensor, Fingerprint portal, traffic/home modules |
| Production telemetry | Same site_id used for AP/client observation cycles; AP-1/AP-2 EAP613 observed |
| Physical scope | VLAN20 / guest subnet / RSPAN path proven independently |

Verdict: CAPTURE_SCOPE_TO_SITE_PROVEN=YES for current production scope. The binding is not based on a single env variable; it is corroborated across runtime configuration, product modules and production telemetry.

# 14. ResourceBudgetBaseline

## 14.1 Fingerprint + Suricata 60-second service interval

| Service | CPU delta / 60 s | Approx one-core utilization | MemoryCurrent | Tasks |
| --- | --- | --- | --- | --- |
| fingerprint-sensor | 0.502 s | ~0.84% | ~45.1 MiB | 5 |
| fingerprint-suricata | 0.458 s | ~0.76% | ~28.6–28.8 MiB | 6 |
| Combined | ~0.960 s | ~1.60% of one core (~0.40% of total 4-vCPU capacity) | ~74 MiB | 11 |

## 14.2 VM headroom

| Resource | Observed |
| --- | --- |
| vCPU | 4 |
| Load average | 3.60 / 3.46 / 3.10 at snapshot; not equivalent to CPU saturation |
| RAM | 7.7 GiB total; ~1.8 GiB used; ~5.6 GiB available |
| Swap | 4.0 GiB total; ~811 MiB allocated; vmstat si=0/so=0 during sample |
| Root filesystem | 98G total; 54G used; ~40G available; 58% used |
| vmstat interval | ~16.5% user, 11.0% system, 61.4% idle, 10.9% I/O wait; avg runnable ~1.18; avg blocked ~0.64 |
| Interpretation | CPU saturation not observed; active swap pressure not observed; moderate I/O wait is present |

# 15. Storage Baseline

| Storage item | Measured result |
| --- | --- |
| Fingerprint sensor directory | ~90 MiB |
| spool.sqlite3 physical size | 93,708,288 bytes (~89.37 MiB) |
| SQLite page_size / page_count | 4096 / 22,878 |
| SQLite freelist_count | 22,872 pages |
| Approx pages not on freelist | 6 pages ≈ 24 KiB |
| Current spool rows from telemetry | 0–6 in observed samples |
| CaptivPortal logs | ~1.3 GiB |
| Log composition | Mostly rotated application logs around 50 MiB each: auth_telemetry, pending_session_cleaner, omada_webhook* |
| Current disk pressure | NOT OBSERVED |
| Future DTI flow/DNS storage | MODELED ONLY — outputs are not currently enabled, so no honest measured growth baseline exists |

# 16. Natural Traffic / Protocol Sample

A bounded 30 s passive sample on enp8s0 observed 672,803 IPv4-formatted lines, 130 IPv6-formatted lines, 262 ICMP and 51 ARP. The attempted awk TCP/UDP classification did not match tcpdump -q formatting, therefore TCP/UDP counts from that command are UNKNOWN rather than zero. Suricata production stats expose capture health but not protocol-mix counters in the current Fingerprint telemetry. The block is therefore PARTIAL and sufficient for DTI-00; no further visitor-traffic probing was justified.

# 17. ControlledClientTraceProof — Owner Decision

Owner-deferred: The controlled owner-phone trace was intentionally not executed at this stage because existing passive/configuration evidence already proves guest MAC/private IPv4 visibility, both directions and pre-NAT placement. It will be reopened only if a later architecture or QA gate requires identity-correlated evidence that cannot be obtained from existing evidence.

Formal verdict: CONTROLLED_CLIENT_TRACE=PARTIAL. This is a deliberate research-scope decision, not a technical failure.

# 18. Fingerprint / Suricata Coexistence Decision

The accepted architecture defines three possible coexistence topologies: A) reuse existing Suricata with an additional versioned DTI output proven not to affect Fingerprint, B) separate DTI Suricata instance, C) another proved safe fan-out. Current evidence strongly favors A operationally because the existing Suricata already captures the mirror, supports required metadata families and has low current resource cost. However A is not formally “proven not to affect Fingerprint” because DTI-00 correctly did not enable new production outputs.

| Option | Assessment |
| --- | --- |
| A — reuse existing Suricata | PREFERRED CANDIDATE. Lowest operational duplication; capabilities present; current resource cost low. Admission still requires controlled compatibility test of additional versioned output. |
| B — separate DTI Suricata | Not justified by current evidence; would add another full-traffic parser and duplicate capture/CPU/memory/operational burden. |
| C — other fan-out | No concrete safer/better arrangement proven during DTI-00. |

FINGERPRINT_SURICATA_COEXISTENCE_RECOMMENDATION = UNRESOLVED
PREFERRED_NEXT_CANDIDATE = A

# 19. Traffic Accounting Source Decision

Current evidence is sufficient to reject the Fingerprint raw parser as a general traffic meter: its BPF is intentionally limited to DHCP and guest outbound TCP SYN, and its semantics are frozen. Suricata supports flow events and directional packet/byte counters, but DTI-00 did not enable flow output and did not prove the periodic/current-rate/visit-boundary behavior required by the accepted DTI architecture.

| Candidate | DTI-00 result |
| --- | --- |
| Fingerprint raw | NOT SUITABLE AS-IS for general accounting; intentionally filtered and specialized. |
| Suricata flow | STRONG CANDIDATE for flow lifecycle/metadata; terminal-flow semantics may be insufficient for periodic rate/current activity/accurate Visit slicing/long-lived accounting. |
| Lightweight traffic meter | NOT YET PROVEN NECESSARY, but remains an explicitly valid candidate for bounded periodic normalized traffic slices if Suricata terminal flow events are insufficient. |
| NetFlow | DIAGNOSTIC / aggregate cross-check only; not authoritative device identity/accounting. |
| Zeek | NO baseline requirement; no concrete gap found. |

TRAFFIC_ACCOUNTING_SOURCE_RECOMMENDATION = UNRESOLVED
PREFERRED RESEARCH DIRECTION = Suricata for general metadata + validate whether a lightweight periodic meter is needed for accounting semantics

# 20. Zeek Gap Analysis

No concrete capability gap was identified that requires a second heavy full-traffic analyzer. Existing custom Fingerprint + Suricata + a possible lightweight accounting collector cover the current architecture needs under investigation. “Zeek can do more” is not a valid admission reason.

ZEEK_REQUIRED = NO

# 21. Parser Allocation Recommendation

| Fact family | Recommended producer | Evidence / rationale |
| --- | --- | --- |
| Fingerprint DHCP | Existing Fingerprint raw | Frozen BPF + DHCP parser already accepted in production |
| Fingerprint TCP SYN | Existing Fingerprint raw | Frozen guest outbound SYN parser already accepted |
| TLS fingerprint / JA4 | Existing Suricata → Fingerprint | Production-enabled and normalized |
| QUIC fingerprint / JA4 | Existing Suricata → Fingerprint | Production-enabled and normalized |
| Capture health | Suricata stats + NIC counters | Kernel/parser/NIC evidence already available |
| DNS metadata | Suricata (future versioned DTI output) | Installed version supports DNS; not currently enabled in Fingerprint config |
| TLS/QUIC general metadata | Suricata | Natural general metadata role; do not duplicate parser |
| Flow lifecycle | Suricata candidate | Installed flow capability; output not currently enabled |
| Periodic bytes/rate/current activity | UNRESOLVED: validate Suricata vs lightweight meter | Architecture requires periodic/Visit-aware semantics; terminal flow events may be insufficient |
| Security alerts | Suricata only if future DTI security scope admits them | Not part of current accounting baseline |
| NetFlow | Diagnostic cross-check | Different observation/identity semantics |
| Zeek | None | No justified gap |

# 22. Resource and Evidence Degradation Policy Recommendation

| Condition | Required behavior |
| --- | --- |
| Capture drops/misses rise | Mark CaptureQuality degraded/partial; do not present traffic accounting as complete/authoritative. |
| Suricata/EVE unavailable or truncated | Fingerprint raw DHCP/SYN remains independent; DTI Suricata-derived facts become unavailable/stale; never convert missing to zero. |
| CPU/RAM/I/O pressure exceeds admitted budget | DTI loses/degrades its own observations first. Portal/Auth/CAPPORT/Omada/Fingerprint are protected. |
| Queue/storage pressure | Bounded drop/reject + telemetry; no unbounded queue growth; no persistent full packet archive. |
| Mirror duplication present | Traffic byte/packet accounting is non-authoritative until remediation/duplicate semantics are frozen. |
| Site attribution ambiguous | Do not guess site_id; do not enter Site-specific per-device results. |
| Source outage | Historical retained summaries may remain readable with provenance; current DTI status becomes stale/unavailable/insufficient. |

# 23. DTICaptureCapabilityMatrix

| Capability | Status | Evidence | Limitation / note |
| --- | --- | --- | --- |
| client MAC | AVAILABLE | E12/E14 | Visible pre-NAT at capture point. |
| client IPv4 | AVAILABLE | E12/E14/E17 | Guest private IPv4 visible. |
| client IPv6 | PARTIAL | E35 | IPv6 observed, not identity-correlated to owner client. |
| VLAN identity | PARTIAL | E02/E03/E12/E15 | Source VLAN20 scope proven; original VLAN tag not preserved at VM API. |
| Site attribution | AVAILABLE | E32 | zefer-span-01 → Site 6a64… → guest CIDR proven. |
| upload direction | AVAILABLE | E14 | Client→remote observed. |
| download direction | AVAILABLE | E14 | Remote→client observed. |
| pre-NAT identity | AVAILABLE | E12/E17/E18 | Capture before Cisco NAT. |
| both flow directions | AVAILABLE | E14 | Bidirectional visibility observed. |
| bytes | UNRELIABLE | E13/E15/E29 | Current RSPAN duplication distorts raw accounting. |
| packets | UNRELIABLE | E13/E15/E29 | Current RSPAN duplication distorts raw counts. |
| flow count | PARTIAL | E30 | Suricata supports flow; production output not enabled; duplicate-safe/current semantics unproven. |
| TCP visibility | AVAILABLE | E12/E28 | Observed; Fingerprint raw has SYN subset. |
| UDP visibility | AVAILABLE | E12/E21 | Observed including NetFlow delivery. |
| ARP | AVAILABLE | E35 | Observed in bounded sample. |
| DHCP | AVAILABLE | E28 | Fingerprint raw filter/parser explicitly covers DHCP. |
| DNS | PARTIAL | E30 | Suricata supports DNS; not currently enabled in Fingerprint EVE. |
| TLS metadata | AVAILABLE | E26/E27 | Production TLS/JA4 path active. |
| QUIC metadata | AVAILABLE | E26/E27 | Production QUIC/JA4 path active. |
| ICMP | AVAILABLE | E35 | Observed in bounded sample. |
| mDNS | UNKNOWN | — | Not specifically validated; no extra probing justified. |
| SSDP | UNKNOWN | — | Not specifically validated; no extra probing justified. |
| capture drop evidence | AVAILABLE | E23/E24/E29 | NIC + Suricata counters available. |
| mirror oversubscription evidence | PARTIAL | E24 | Current rate/headroom measured; source/destination reconciliation not complete. |
| parser drop evidence | AVAILABLE | E29 | Suricata kernel drops and EVE health telemetry. |
| event timestamps | AVAILABLE | E27/E29 | Kernel timestamps + EVE timestamps/telemetry. |
| host capture path | AVAILABLE | E04–E11 | End-to-end host path proven. |
| controlled client correlation | PARTIAL | E36 | Owner-deferred as redundant; reopen if later required. |
| NetFlow cross-check | PARTIAL | E19–E21 | Exporter/delivery proven; .202 collector blocked by firewall; .222 PRTG excluded from deeper DTI scope. |

# 24. Unknowns, Limitations and Non-Claims

Exact RSPAN duplication root cause was not finalized by configuration change. Strongest candidate: VLAN10+VLAN20 + BOTH around router-on-a-stick. Repair is post-report work.

Absolute mirror completeness is not proven. Current bounded quality evidence is good; oversubscription remains POSSIBLE.

Exact byte semantics for DTI (L2 vs IP vs transport etc.) are not frozen by this task.

Periodic accounting semantics required for current rate/Visit slicing/long-lived flows remain unresolved; Suricata terminal flow capability alone is not sufficient proof.

IPv6 was observed but no controlled IPv6 client correlation was performed.

mDNS/SSDP exact coverage was not separately validated.

Upstream public-NAT placement after Cisco3845 was not required and is not claimed.

The 52.29% Fingerprint duplicate ratio applies only to the frozen raw BPF subset, not to all mirrored traffic.

# 25. Changes Made During Research

| Change | Authorization / impact |
| --- | --- |
| Install tcpdump on omv-hp | Explicit separate Owner authorization for bounded DTI-00 diagnostic evidence. tcpdump 4.99.3 / libpcap 1.10.3 observed after install. |
| SPAN/RSPAN configuration | NONE |
| Switch VLAN/trunk configuration | NONE |
| Cisco routing/NAT/ACL | NONE |
| Omada configuration | NONE |
| Fingerprint service/config | NONE |
| Suricata production config/output | NONE |
| Firewall policy | NONE |
| NIC offload/ring tuning | NONE |

# 26. Mandatory Post-DTI-00 Follow-up

Required Owner instruction: After this report is delivered and reviewed by Architect + Tech Lead, the RSPAN packet-duplication defect must be corrected and removed. This is mandatory follow-up, not optional cleanup.

Determine approved correction for the current Catalyst RSPAN topology; do not guess the fix from DTI-00 alone.

Remove the duplicate-capture condition.

Validate that VLAN20 evidence remains visible and both client→remote and remote→client directions are preserved.

Validate Fingerprint raw DHCP/TCP-SYN behavior remains correct and dedup ratio collapses to expected background/retransmission behavior rather than mirror duplication.

Validate Suricata remains healthy and capture/kernel drop counters do not regress.

Repeat bounded physical + VM capture to confirm duplicate removal.

Only after duplicate remediation, run the specific accounting admission experiment needed to decide Suricata flow vs lightweight periodic meter.

Update this report with a remediation addendum/status; preserve the original finding rather than rewriting history.

Optional separate issue: if 192.168.0.202:9995 NetFlow remains required, admit UDP/9995 through host firewall under its own reviewed change and revalidate nfcapd ingest.

Optional tuning candidate: evaluate RX ring increase from 256 toward supported 4096 only after review/benchmark; historical misses alone are not authorization to tune.

# 27. Final DTI-00 Verdict

| Field | Value | Note |
| --- | --- | --- |
| TASK | TASK-DTI-00 |  |
| MODE | RESEARCH / LAB / INVENTORY |  |
| CAPTURE_TOPOLOGY_PROVEN | YES |  |
| CAPTURE_PLACEMENT_PROVEN | YES |  |
| HOST_CAPTURE_PATH_PROVEN | YES |  |
| CONTROLLED_CLIENT_TRACE | PARTIAL | Owner-deferred as redundant at current stage |
| CLIENT_MAC_VISIBILITY | YES |  |
| VLAN_VISIBILITY | PARTIAL | Source VLAN scope proven; original VLAN tag not retained at VM capture presentation |
| PRE_NAT_VISIBILITY | YES |  |
| BOTH_FLOW_DIRECTIONS | YES |  |
| CAPTURE_PRESENTATION_EFFECTS | ACCOUNTED |  |
| DUPLICATE_RISK | CONFIRMED |  |
| MIRROR_OVERSUBSCRIPTION_RISK | POSSIBLE |  |
| CAPTURE_COMPLETENESS_EVIDENCE | PARTIAL |  |
| CAPTURE_SCOPE_TO_SITE_PROVEN | YES |  |
| CAPTURE_QUALITY_BASELINE | PASS |  |
| RESOURCE_BASELINE | PASS |  |
| FINGERPRINT_SURICATA_COEXISTENCE_RECOMMENDATION | UNRESOLVED | Option A is preferred candidate but additional DTI output was not admitted/tested under DTI-00 |
| TRAFFIC_ACCOUNTING_SOURCE_RECOMMENDATION | UNRESOLVED | Suricata is preferred metadata/flow candidate; lightweight periodic meter remains candidate for rate/Visit semantics |
| ZEEK_REQUIRED | NO |  |
| NETFLOW_ROLE | DIAGNOSTIC |  |
| RESEARCH_PRIVACY_BOUNDARY | PASS |  |
| OPEN_P0 | 0 |  |
| OPEN_P1 | 1 | RSPAN packet duplication blocks authoritative packet/byte accounting until remediation |
| BLOCKERS | 3 items | 1) Formal full PASS lacks ControlledClientTrace by explicit Owner decision. 2) Coexistence/accounting admission requires separate approved validation. 3) Authoritative raw accounting awaits duplication remediation. |
| CHANGES_MADE | tcpdump diagnostic install only | Explicit Owner authorization; no production network/parser/firewall configuration changed. |
| DTI_00_VERDICT | PARTIAL | Evidence-grade verdict; physical/host baseline substantially proven. |

Interpretation: PARTIAL is the correct evidence-grade verdict. The physical/host capture baseline is substantially proven and the research achieved its main purpose, including discovery of defects not known at task start. PARTIAL prevents the report from falsely certifying an owner-client trace, duplicate-free accounting, or an untested production-output topology.

# Appendix A — Canonical Evidence Index

| ID | Host / source | Command / observation | Supports conclusion | Class |
| --- | --- | --- | --- | --- |
| E01 | Cisco2960 | show version / clock | Catalyst identity: WS-C2960S-48LPS-L, IOS 15.2(2)E9, serial FOC1846Z2UF. | OBSERVED |
| E02 | Cisco2960 | show monitor session all; show run ¦ section ^monitor | RSPAN session 1; source VLAN10,20 BOTH; remote destination VLAN999. | CONFIGURATION-PROVEN |
| E03 | Cisco2960 | show vlan id 999; show interfaces trunk | VLAN999 RSPAN enabled; transported on Gi1/0/48 trunk, native40, allowed 40,999. | CONFIGURATION-PROVEN |
| E04 | Cisco2960 + omv-hp | MAC table + ip link | Gi1/0/48 learned 44:1e:a1:61:e8:d9; exact match enp7s0f1. | OBSERVED |
| E05 | omv-hp | ip -d link | enp7s0f1.999 VLAN999; macvtap0 passthru. | CONFIGURATION-PROVEN |
| E06 | omv-hp/libvirt | virsh domiflist | Ubunt VM direct macvtap0 / enp7s0f1.999 MAC 52:54:00:59:23:b8. | CONFIGURATION-PROVEN |
| E07 | Ubuntu VM | ip/link/address observation | enp1s0=192.168.0.202; enp8s0 capture NIC no IP; MAC matches macvtap. | OBSERVED |
| E08 | Ubuntu VM | systemctl/process inventory | Fingerprint classification/evidence/sensor/Suricata services running. | OBSERVED |
| E09 | Ubuntu VM | systemctl show/cat fingerprint-suricata | Suricata 8.0.6 AF_PACKET enp8s0 workers; EVE unix socket; resource limits observed. | CONFIGURATION-PROVEN |
| E10 | Ubuntu VM | sensor.env + ss -xap | capture_source=zefer-span-01; site/cidr configured; EVE unix socket held by sensor. | CONFIGURATION-PROVEN |
| E11 | Ubuntu VM | /proc/net/packet + FD mapping | Two direct packet consumers: fingerprint-sensor + Suricata. | OBSERVED |
| E12 | Ubuntu VM | bounded tcpdump enp8s0 | Guest MAC/private IPv4 and pre-NAT traffic visible; VM capture API untagged. | OBSERVED |
| E13 | Ubuntu VM | bounded packet identity comparison | Exact duplicated packets observed at enp8s0. | OBSERVED |
| E14 | Ubuntu VM | guest-filtered tcpdump | Both outbound and return guest traffic observed. | OBSERVED |
| E15 | omv-hp | tcpdump enp7s0f1 vlan999 and enp7s0f1.999 | Duplicates already present on physical RSPAN ingress; VLAN999 tag visible physically. | OBSERVED |
| E16 | Cisco3845 | show version / interfaces | Cisco3845 identity and interface inventory. | OBSERVED |
| E17 | Cisco3845 | show run/int Gi0/1.20 | Guest first L3 192.168.10.1/22, dot1Q20, NAT inside. | CONFIGURATION-PROVEN |
| E18 | Cisco3845 | Gi0/0 + show ip nat statistics/config | NAT outside on Gi0/0 192.168.100.10; overload ACL100. Capture pre-Cisco-NAT. | CONFIGURATION-PROVEN |
| E19 | Cisco3845 | show ip flow export / config | NetFlow v9 destinations .222:2055 and .202:9995; differs from earlier .220 expectation. | CONFIGURATION-PROVEN |
| E20 | Ubuntu VM | ss/systemctl/cat nfcapd/nfdump | nfcapd :9995 → /var/nfdump; second nfcapd :2055 → /var/cache/nfdump; historical storage measured. | CONFIGURATION-PROVEN |
| E21 | Ubuntu VM | nfdump/journal + tcpdump + nftables | UDP/9995 reaches enp1s0; nfcapd sees 0 flows; nftables INPUT DROP has no UDP/9995 allow. | CONFIGURATION-PROVEN |
| E22 | omv-hp | ethtool -i/-k + lspci | Intel 82576 / HP NC362i, igb, firmware 1.8.2; 1Gb/full; offloads inventoried. | OBSERVED |
| E23 | omv-hp | ethtool -S | Historical rx_missed/fifo=46121, no_buffer=2074, CRC=1; queue drops zero. | OBSERVED |
| E24 | omv-hp | 60 s before/after counters | ~255.9 Mb/s physical, ~156.6 Mb/s VLAN999; no new miss/fifo/no-buffer/CRC/VLAN999 drop. | OBSERVED |
| E25 | omv-hp | ethtool -g | RX ring current 256, max 4096; TX same. | CONFIGURATION-PROVEN |
| E26 | Ubuntu VM | production suricata.yaml | Current EVE: TLS custom JA4/ALPN/handshake, QUIC, stats only. | CONFIGURATION-PROVEN |
| E27 | Ubuntu VM | Fingerprint normalizer/runtime source inspection | TLS/QUIC/stats EVE are consumed and normalized; Suricata heartbeat/health semantics present. | CONFIGURATION-PROVEN |
| E28 | Ubuntu VM | capture_filter.py + capture.py + dedup.py | Raw BPF = DHCP + guest outbound SYN; exact BLAKE2s dedup 0.25 ms / 65536 before normalizer. | CONFIGURATION-PROVEN |
| E29 | Ubuntu VM | journal fingerprint metrics/stats | raw_dedup_ratio ~52.29% filtered subset; Suricata capture_kernel_drops=4098 stable during sample; runtime ready. | OBSERVED |
| E30 | Ubuntu VM | suricata --build-info + stock /etc/suricata/suricata.yaml | Installed Suricata supports AF_PACKET, flow, DNS, DHCP, HTTP, TLS, QUIC, stats, alert; not all enabled in production. | CONFIGURATION-PROVEN |
| E31 | Ubuntu VM | 60 s systemctl CPUUsageNSec/MemoryCurrent | Sensor ~0.502 CPU-s; Suricata ~0.458 CPU-s; combined ~74 MiB memory. | OBSERVED |
| E32 | Ubuntu VM | site/capture grep across runtime/docs/logs | zefer-span-01 + site 6a64… + 192.168.8.0/22 consistently bound; same Site in production telemetry. | CONFIGURATION-PROVEN |
| E33 | Ubuntu VM | nproc/uptime/free/df + vmstat 5 12 | 4 vCPU; RAM/disk headroom good; no active swap; ~61% avg idle, ~11% I/O wait. | OBSERVED |
| E34 | Ubuntu VM | du + SQLite read-only PRAGMA + log file inventory | Spool physical ~89.4 MiB but 22,872/22,878 pages free; logs 1.3G mostly rotated app logs. | OBSERVED |
| E35 | Ubuntu VM | 30 s bounded tcpdump protocol sample | IPv4 dominant; IPv6/ICMP/ARP observed; TCP/UDP awk classification inconclusive. | OBSERVED / PARTIAL |
| E36 | Owner decision | Explicit scope decision | Controlled owner-client trace deferred as redundant; reopen only if future gate requires identity-correlation. | OWNER-DEFERRED |
| E37 | omv-hp | apt install tcpdump under explicit authorization | tcpdump installed solely for bounded DTI evidence; separate Owner-approved diagnostic change. | CHANGE RECORDED |
| E38 | Owner scope decision | Owner confirmation | 192.168.0.222:2055 is PRTG; deeper collector research excluded as unrelated to primary DTI path. | OWNER-CONFIRMED |
| E39 | Cross-evidence synthesis | E02/E15 + router-on-a-stick topology | Likely duplicate mechanism is multi-VLAN/BOTH mirroring around routed traffic; exact root cause pending post-report corrective test. | STRONGLY SUPPORTED / NOT FINAL |
| E40 | Architecture synthesis | Fingerprint + Suricata + accepted DTI architecture | Fingerprint remains specialized; Suricata general metadata role; periodic accounting source still requires admission proof. | RECOMMENDATION |
| E41 | Owner instruction | Explicit post-DTI requirement | After report/review, duplication must be fixed and full post-fix validation performed; update report with remediation status. | MANDATORY FOLLOW-UP |

# Appendix B — Key Command / Observation Inventory

| Host | Primary read-only evidence commands |
| --- | --- |
| Cisco2960 | show monitor session all; show run ¦ section ^monitor; show vlan id 999; show interfaces trunk; MAC table; interface status/counters |
| Cisco3845 | show version; show interfaces; show run interface Gi0/1.20; Gi0/0; show ip nat statistics; show ip flow export; show run ¦ include ip flow-export |
| omv-hp | ip -d link; virsh domiflist; tcpdump bounded captures; ethtool -i/-k/-S/-g enp7s0f1; lspci -s 07:00.1 -nnk; 60 s counter delta |
| Ubuntu VM | ip/link/address; systemctl show/cat; ss; /proc/net/packet; process FD mapping; tcpdump bounded captures; nftables/iptables; nfdump/nfcapd; source inspection under /opt/CaptivePortal/app/device_fingerprint_sensor; Suricata build info/config; systemctl resource deltas; vmstat; SQLite read-only PRAGMAs; storage/log inventory |

# Appendix C — Questions for Architect / Tech Lead Review

1. Approve the post-DTI RSPAN duplication remediation experiment and define the allowed switch change/test window.

2. Confirm whether Option A (reuse existing Suricata with an isolated/versioned DTI output) should be the first coexistence admission test, or whether isolation requirements force B/C.

3. For periodic traffic accounting, define the exact evidence needed to decide Suricata flow vs lightweight meter: cadence, long-lived flow treatment, Visit slicing, byte basis, retransmission/dedup semantics.

4. Confirm whether .202:9995 NetFlow is still required. If yes, authorize a separate firewall correction and revalidation; if no, retire/ignore it rather than repairing unused infrastructure.

5. Decide whether the historical NIC misses justify a controlled RX-ring benchmark after duplication remediation; no tuning was performed in DTI-00.

6. Accept CONTROLLED_CLIENT_TRACE=PARTIAL/Owner-deferred for this report, or explicitly request one targeted trace if formal PASS is required before DTI-01.

END OF REPORT