TASK-DTI-00

CAPABILITY & CAPTURE BASELINE REPORT

CaptivPortal — Device Traffic Enrichment / Device Intelligence

Research / Lab / Inventory
FINAL POST-REMEDIATION HANDOFF

Evidence date: 2026-10-05 (AZST / UTC+4)
Audience: Owner, Architect, Tech Lead
Report status: final research + remediation handoff for review

# Document Control

| Field | Value |
| --- | --- |
| Project | CaptivPortal |
| Program | Device Traffic Enrichment / Device Intelligence (DTI) |
| Task | TASK-DTI-00 — Capability & Capture Baseline |
| Mode | RESEARCH / LAB / INVENTORY |
| Implementation authorization | NO |
| Production feature activation | NO |
| Primary live evidence date | 2026-10-05 |
| Status | FOR ARCHITECT + TECH LEAD REVIEW — final post-remediation |
| Primary contract | TASK-DTI-00-CAPABILITY-CAPTURE-BASELINE-SYSADMIN-FINAL.md |
| Architecture reference | DEVICE-TRAFFIC-ENRICHMENT-ARCHITECTURE-v0.3_FINAL_TECHLEAD_ACCEPTED.md |
| Post-research remediation date | 2026-10-05 |

Canonical evidence numbering: This final report canonicalizes Evidence IDs E01–E48. E01–E41 preserve the research baseline; E42–E48 record the post-research RSPAN remediation and validation. Interim chat labels were working labels and are not authoritative.

# 1. Executive Summary

TASK-DTI-00 established the actual production mirror/capture path, placement, software consumers, resource characteristics and limitations, and discovered a major production RSPAN duplication defect that was not assumed at task start. The research phase itself did not implement DTI and did not alter production SPAN/RSPAN, VLAN, routing, NAT, Fingerprint or Suricata configuration. After the evidence freeze, the Owner explicitly authorized a targeted post-research remediation: the Catalyst VLAN-based RSPAN source direction was changed from BOTH to RX-only while retaining source VLAN10+VLAN20 and destination RSPAN VLAN999. The systematic ~50% exact-duplicate pattern disappeared from the Fingerprint raw candidate stream, VLAN20 remained visible in both directions, Fingerprint and Suricata stayed healthy, capture drops did not increase, and the RX-only setting was persisted to startup-config. The final DTI-00 verdict remains PARTIAL only because the controlled owner-client trace was intentionally deferred and the Suricata-coexistence / authoritative periodic accounting admission decisions remain unresolved; the original RSPAN duplication P1 is CLOSED.

| Area | Result |
| --- | --- |
| Capture topology | PROVEN end-to-end: Catalyst RSPAN → omv-hp → VLAN999/macvtap → Ubuntu VM enp8s0. |
| Placement | PROVEN pre-Cisco-NAT; client MAC/private IPv4 and both traffic directions are visible. |
| RSPAN defect | FOUND AND REMEDIATED. Baseline systematic duplication was already present at physical RSPAN ingress. Post-research fix: source VLAN10,20 direction BOTH → RX-only; persisted to startup-config and validated. |
| Fingerprint protection | PROVEN exact-frame dedup (0.25 ms, 65,536-entry bounded cache) before raw DHCP/TCP-SYN normalization. |
| Live duplicate impact | Baseline: ~52.29% cumulative duplicate ratio in the Fingerprint raw filtered candidate stream. Post-fix validation: raw_packet_count +1,345 with duplicate_packet_count +0 from 14:19 to 14:30 AZST. Baseline ratio MUST NOT be extrapolated to all VLAN999 traffic. |
| Capture quality | Current 60 s sample clean: no new CRC/no-buffer/missed/fifo/VLAN999 drops; historical receive loss exists. |
| Current mirror load | ~156.6 Mbit/s VLAN999 average during measured 60 s; physical NIC total RX ~255.9 Mbit/s. |
| Suricata | 8.0.6; production emits TLS/QUIC/stats; installed version supports flow/DNS/DHCP/HTTP and other EVE families. |
| Resource headroom | Current Fingerprint+Suricata ~0.96 CPU-seconds/60 s (~1.6% of one core) and ~74 MiB RAM; VM has substantial RAM/CPU headroom with moderate I/O wait. |
| Storage | Fingerprint spool physical ~89.4 MiB but ~89.34 MiB is SQLite freelist; active queue footprint negligible at sample time. |
| Site binding | PROVEN: zefer-span-01 → site 6a64f17630da7c70d232187a → guest 192.168.8.0/22. |
| NetFlow | Diagnostic only. Cisco exports to .222:2055 and .202:9995; .202:9995 packets arrive but are dropped by host nftables before nfcapd. |
| Zeek | NO baseline justification; no concrete gap established. |
| Post-research remediation | COMPLETED: RX-only RSPAN source validated for VLAN20 upload/download, physical capture, Fingerprint/Suricata health and stable drop counters. Remaining next work is architecture review and accounting-source admission testing. |

Important decision refinement: The interim hypothesis “Suricata-only traffic accounting” is not treated as a final proven decision. The accepted architecture explicitly warns that terminal flow events may be insufficient for current rate, current activity, visit-boundary slicing and long-lived-flow accounting. Final DTI-00 position: Suricata is the preferred general metadata engine; authoritative periodic traffic accounting source remains UNRESOLVED pending the next approved validation.

# 2. Scope, Method and Safety Boundary

The research object was the currently operational production SPAN/RSPAN / Device Fingerprint capture infrastructure. Future server designs were explicitly excluded as current facts. Evidence was classified as OBSERVED, CONFIGURATION-PROVEN, CONTROLLED-TRACE-PROVEN, MODELED, UNKNOWN, or NOT_APPLICABLE.

Preferred mode: read-only inspection + bounded passive observation.

During the research evidence-collection phase, no production SPAN/RSPAN, VLAN, trunk, routing, NAT, Omada, Fingerprint, Suricata or firewall configuration was changed. After evidence freeze, the Owner explicitly authorized one post-research corrective change: RSPAN source direction BOTH → RX-only for VLAN10+VLAN20; no VLAN membership, trunk, routing, NAT, Fingerprint, Suricata or firewall settings were changed.

No persistent full PCAP or arbitrary visitor payload was collected.

Two explicit Owner-authorized exceptions are recorded: (1) tcpdump was installed on omv-hp solely for bounded diagnostic evidence collection; (2) after the research report was produced, the Catalyst RSPAN source direction was changed from BOTH to RX-only to remediate the discovered systematic duplication defect and then persisted after validation.

Owner-controlled client trace was intentionally deferred after the existing evidence had already proved MAC/IP visibility, bidirectionality and pre-NAT placement. Re-open only if a later decision requires identity-correlated proof not available from existing evidence.

# 3. Current Production Capture Topology

Guest client / Zefer_Parki
        | VLAN20 (192.168.8.0/22)
        v
EAP613 AP (observed AP attachment on Catalyst Gi1/0/10)
        | 802.1Q trunk; native VLAN10; allowed 10,20,50
        v
Cisco2960 WS-C2960S-48LPS-L
        |
        +--> ORIGINAL TRAFFIC: Gi1/0/1 trunk --> Cisco3845 Gi0/1.20
        |       gateway 192.168.10.1/22, MAC 00:24:97:7b:b9:41
        |       ip nat inside
        |       --> Gi0/0 192.168.100.10 (ip nat outside) --> upstream
        |
        +--> RSPAN session 1
                source VLANs: 10,20  direction: RX ONLY (final; baseline was BOTH)
                destination remote VLAN: 999 (RSPAN_DEVICE_FP)
                |
                v
             Gi1/0/48 trunk (native 40; allowed 40,999; 1Gb/full)
                |
                v
             omv-hp / enp7s0f1 (Intel 82576 / HP NC362i)
                | vlan999 tagged on physical capture API
                v
             enp7s0f1.999 --> macvtap0 (passthru)
                |
                v
             VM "Ubunt" / local.domain.az
                |
                +--> enp1s0 = 192.168.0.202 management/application
                |
                +--> enp8s0 = capture NIC, no IP
                       |
                       +--> fingerprint-sensor AF_PACKET + frozen BPF + exact dedup
                       |      DHCP + guest outbound TCP SYN
                       |
                       +--> Suricata 8.0.6 AF_PACKET workers
                              --> EVE unix_dgram socket
                              --> fingerprint-sensor TLS/QUIC/stats normalization

## 3.1 Switch and RSPAN facts

| Item | Observed/configured value |
| --- | --- |
| Catalyst | Cisco2960; WS-C2960S-48LPS-L; IOS 15.2(2)E9; serial FOC1846Z2UF |
| RSPAN session | Session 1, Remote Source Session |
| Source scope | VLAN10 + VLAN20, RX Only (final post-remediation; baseline was Both) |
| RSPAN VLAN | 999 / RSPAN_DEVICE_FP / Remote SPAN VLAN Enabled |
| Destination transport | Gi1/0/48; 802.1Q trunk; native VLAN40; allowed/active/forwarding VLAN40,999; 1Gb/full |
| Guest AP port | Gi1/0/10, description AP - EAP613 NEW; trunk native VLAN10; allowed 10,20,50 |
| Gateway-facing port | Gi1/0/1 trunk; allowed 10,20,30,50 |
| Router gateway MAC | 0024.977b.b941 learned on Gi1/0/1 for VLAN10/20/30 |

# 4. HostCapturePathProof

The physical switch destination and the VM capture interface were independently correlated by MAC addresses and libvirt topology. This proves the complete host-side delivery path rather than assuming a historic interface name.

| Layer | Evidence |
| --- | --- |
| Catalyst Gi1/0/48 | VLAN40 MAC 44:1e:a1:61:e8:d9 learned on port |
| omv-hp physical NIC | enp7s0f1 MAC 44:1e:a1:61:e8:d9 — exact match |
| RSPAN subinterface | enp7s0f1.999, VLAN protocol 802.1Q id 999, UP |
| macvtap | macvtap0@enp7s0f1.999, passthru, MAC 52:54:00:59:23:b8 |
| VM libvirt mapping | Ubunt direct macvtap0 / enp7s0f1.999, virtio MAC 52:54:00:59:23:b8 |
| VM capture NIC | enp8s0 MAC 52:54:00:59:23:b8, UP, no IP |
| VM management NIC | enp1s0 MAC 52:54:00:0d:2f:54, 192.168.0.202/24 |

# 5. Capture Placement, Identity and Directionality

Guest client private IPv4 and client Ethernet MAC are visible at enp8s0.

Both client→remote and remote→client traffic were passively observed.

Original guest VLAN tag is not visible at the VM capture API; the RSPAN transport VLAN999 tag is visible on the physical host NIC and is removed by the Linux .999 subinterface as expected.

Capture is conclusively before Cisco3845 NAT. The router guest subinterface is Gi0/1.20 = 192.168.10.1/22 with ip nat inside; Gi0/0 = 192.168.100.10/24 with ip nat outside.

The upstream public-NAT location beyond Cisco3845 was not required and was not claimed.

# 6. Mirror Duplication Analysis

Major finding and closure: Exact packet/frame duplication was CONFIRMED and reproducible in the baseline, visible at both VM enp8s0 and omv-hp physical RSPAN ingress. The initial duplication was therefore upstream of macvtap/VM. A post-research one-variable correction (VLAN source direction BOTH → RX-only) removed the systematic duplicate pattern while preserving both VLAN20 traffic directions. The defect is recorded as discovered, remediated and validated; low-rate residual exact-frame repeats in broad physical capture are retained as a separate observation, not rewritten away.

Baseline evidence showed repeated frames with effectively identical timing and packet identity. Some equivalent L3 packets were also visible in different Ethernet contexts, consistent with routed traffic being observed across monitored VLANs. The strongest baseline hypothesis was VLAN10+VLAN20 with direction BOTH in a router-on-a-stick topology. The post-research corrective experiment validated the operational cause of the mass duplicate pattern: only the source direction was changed from BOTH to RX-only; VLAN10, VLAN20 and RSPAN VLAN999 were left unchanged. After the change, Fingerprint metrics advanced from raw_packet_count=153,229 / duplicate_packet_count=80,087 at 14:19 to raw_packet_count=154,574 / duplicate_packet_count=80,087 at 14:30 — +1,345 raw candidates with +0 new exact duplicates. VLAN20 upload and download remained visible. Physical RSPAN samples still contained low-rate exact repeats (10/3,599 = 0.2779%; 2/302 = 0.6623%); these are documented as residual exact-frame occurrences and are not treated as recurrence of the former ~50% systematic RSPAN duplication.

| Aspect | Finding |
| --- | --- |
| Duplication at VM enp8s0 | CONFIRMED |
| Duplication at omv-hp enp7s0f1 VLAN999 | CONFIRMED |
| VM/macvtap as initial source | EXCLUDED by physical-host evidence |
| Operational root cause | Baseline VLAN-based RSPAN source direction BOTH in this topology produced the systematic duplicate pattern; validated by one-variable BOTH → RX correction. |
| Production correction | monitor session 1 source vlan 10 , 20 rx; destination remote vlan 999 unchanged; VLAN10/VLAN20 source scope unchanged. |
| Impact | Before fix: distorted packet/byte/event accounting and unnecessary parser load. After fix: systematic duplicate growth stopped; residual low-rate exact repeats remain a separate accounting-quality consideration. |
| Fingerprint post-fix delta | 14:19→14:30 AZST: raw +1,345; exact duplicates +0; duplicate counter remained 80,087. |
| VLAN20 direction validation | UPLOAD: 41 captured / 53 received by filter / 0 kernel drops. DOWNLOAD: 383 captured / 403 received by filter / 0 kernel drops. |
| Physical post-fix sample #1 | 3,599 packets; 10 exact repeats within 1 ms; ratio 0.002779 (0.2779%); 0 tcpdump kernel drops. |
| Physical post-fix sample #2 | 302 packets; 2 exact repeats within 1 ms; ratio 0.006623 (0.6623%); 0 tcpdump kernel drops. Detailed repeats included different L2 contexts for the same routed L3 direction. |
| Service health after fix | fingerprint-sensor=active; fingerprint-suricata=active; runtime_state=ready; eve_datagram_truncated=0; eve_json_invalid=0; Suricata active. |
| Suricata capture drops | capture_kernel_drops stayed 4,098 while capture_kernel_packets continued increasing; no post-fix drop growth observed in validation window. |
| Persistence | startup-config verified: monitor session 1 source vlan 10 , 20 rx / destination remote vlan 999. |
| Final defect status | SYSTEMATIC_RSPAN_DUPLICATION = REMEDIATED / VALIDATED. Original P1 closed. |

# 7. Existing Fingerprint Sensor: Raw Path, Dedup and Suricata Path

## 7.1 Two parallel packet consumers

Linux /proc/net/packet and process FD mapping proved two independent direct packet consumers on enp8s0: fingerprint-sensor and Suricata. Suricata additionally feeds EVE events back to fingerprint-sensor over a Unix datagram socket.

enp8s0
  +--> fingerprint-sensor raw AF_PACKET socket
  |      --> frozen BPF
  |      --> exact-frame dedup
  |      --> DHCP / TCP-SYN normalization
  |
  +--> Suricata AF_PACKET workers
         --> EVE unix_dgram
         --> fingerprint-sensor TLS / QUIC / stats normalization

## 7.2 Frozen raw-capture filter

(udp and (port 67 or port 68)) or
(src net 192.168.8.0/22 and tcp[tcpflags] & (tcp-syn|tcp-ack) == tcp-syn)

Therefore Fingerprint raw_packet_count is NOT a whole-mirror traffic counter. It covers DHCP and guest outbound TCP SYN candidates only.

## 7.3 Exact-frame deduplication

| Parameter | Value |
| --- | --- |
| Algorithm | BLAKE2s over capture_source_id + frame length + exact frame bytes |
| Digest size | 16 bytes |
| Default horizon | 0.25 ms |
| Maximum cache entries | 65,536 |
| Placement | Before DHCP/TCP-SYN normalization |
| Behavior | Duplicate candidate is dropped from raw normalization path |

Baseline production telemetry showed raw_dedup_ratio ≈ 0.5229. Example cumulative point at 11:49:19 AZST: raw_packet_count=135,955; duplicate_packet_count=71,091; deduplicated_packet_count=64,864. This proved strong duplicate impact in the FILTERED Fingerprint candidate stream only and MUST NOT be extrapolated to all VLAN999 packets. Post-remediation telemetry independently showed the systematic mirror effect had stopped: from 14:19 to 14:30 raw_packet_count increased 153,229 → 154,574 (+1,345) while duplicate_packet_count remained exactly 80,087 (+0). The cumulative raw_dedup_ratio therefore declines slowly because historic duplicates remain in the lifetime counters.

# 8. Suricata Inventory and Capability

| Item | Result |
| --- | --- |
| Version | Suricata 8.0.6 RELEASE |
| Capture mode | AF_PACKET on enp8s0, workers |
| Current EVE transport | unix_dgram → /run/captive-portal/fingerprint-sensor/suricata-eve.sock |
| Current enabled EVE types | TLS (JA4, client_alpns, client_handshake), QUIC, stats |
| Installed-version capabilities observed | flow, DNS, DHCP, HTTP, TLS, QUIC, stats, alert; AF_PACKET; JA4; Hyperscan; Unix socket |
| Not enabled in current Fingerprint config | flow, DNS, HTTP, alert |
| Current Fingerprint consumption | TLS/QUIC normalized to evidence; stats used as Suricata heartbeat/capture health |

Coexistence boundary: The accepted Fingerprint Suricata configuration is protected. DTI-00 did not enable flow/DNS/HTTP or otherwise mutate fingerprint-suricata.service.

# 9. NIC / Kernel Presentation and Capture Quality

| Item | Value |
| --- | --- |
| NIC | Intel 82576 Gigabit Network Connection / HP NC362i integrated dual-port adapter |
| Interface | omv-hp enp7s0f1; PCI 0000:07:00.1 |
| Driver / firmware | igb / firmware 1.8.2 |
| Link | 1000 Mb/s, Full Duplex, autoneg on |
| RX offloads/presentation | rx-checksum ON; GRO ON; LRO OFF; rx-vlan-offload ON; RX hash ON |
| TX/GSO | TSO ON; GSO ON; scatter-gather ON |
| RX ring | current 256; maximum 4096 |
| RSPAN physical presentation | 802.1Q VLAN999 observed at enp7s0f1; untagged at enp7s0f1.999 / VM as expected |

## 9.1 Historical counters

| Counter | Observed cumulative value / interpretation |
| --- | --- |
| rx_crc_errors | 1 |
| rx_no_buffer_count | 2,074 |
| rx_missed_errors | 46,121 |
| rx_fifo_errors | 46,121 |
| per-RX-queue drops | 0 on all observed queues |
| Interpretation | Historical receive loss exists; not proof of current sustained loss |

## 9.2 60-second natural-load interval

| Metric | Physical enp7s0f1 | VLAN999 |
| --- | --- | --- |
| Packets delta | 1,829,066 | 1,151,004 |
| Average packets/s | ~30,484 | ~19,183 |
| Bytes delta | 1,918,930,440 | 1,174,563,011 |
| Average rate | ~255.9 Mbit/s | ~156.6 Mbit/s |
| New rx_missed/fifo/no-buffer/CRC | 0 | — |
| New rx_dropped | — | 0 |

The current sample was clean at substantial natural load. This does not prove lifetime completeness. The physical interface also carries non-RSPAN traffic (native VLAN40), so physical total RX is not equal to VLAN999 traffic.

# 10. Mirror Capacity / Headroom

The destination link is 1 Gb/s full-duplex. Baseline VLAN999 averaged ~156.6 Mb/s in the bounded 60 s sample; physical RX averaged ~255.9 Mb/s. This demonstrated headroom in that sampled window. The final RSPAN source is now VLAN10+VLAN20 RX-only, which removed the previous BOTH-direction systematic duplication and should reduce avoidable mirrored load. Nevertheless, aggregate ingress across all monitored VLAN ports can still in principle exceed a single 1 Gb/s RSPAN destination, and no synchronized post-fix source-side/destination-side reconciliation has been performed. Therefore oversubscription remains POSSIBLE, not proven absent.

Required semantic: CAPTURE_QUALITY_BASELINE=PASS means monitoring/evidence quality is sufficient for the baseline. It does NOT mean 100% packet completeness.

# 11. Cisco3845 / First L3 / NAT

| Item | Observed value |
| --- | --- |
| Router | Cisco3845; IOS 15.1(4)M12a; serial FOC130427S0 |
| Guest first L3 | Gi0/1.20, dot1Q 20, 192.168.10.1/22, MAC 00:24:97:7b:b9:41 |
| Guest ACL | GUEST_TTL_FILTER inbound |
| Flow accounting | ip flow ingress + ip flow egress |
| NAT inside | Gi0/1.10, Gi0/1.20, Gi0/1.30 |
| NAT outside | Gi0/0, 192.168.100.10/24 |
| NAT rule | ip nat inside source list 100 interface Gi0/0 overload |
| Capture relationship | RSPAN sees guest traffic before Cisco NAT |

# 12. NetFlow / nfdump Overlap and Additional Defect

NetFlow was investigated as diagnostic/cross-check evidence, not as the primary device identity source.

| Finding | Evidence / status |
| --- | --- |
| Cisco exporter version | NetFlow v9 |
| Actual current destinations | 192.168.0.222:2055 and 192.168.0.202:9995 |
| Difference from earlier stated expectation | Earlier scope mentioned 192.168.0.220:2055; live router proves .222:2055 + .202:9995 |
| .202:9995 listener | nfcapd PID/service present; /var/nfdump configured; historical directory ~16G |
| .202:9995 current ingest | 0 flows; current nfcapd files ~276 B/header-only |
| Network delivery to .202:9995 | PROVEN: UDP datagrams from 192.168.0.1 reach enp1s0 |
| Root cause of zero ingest | CONFIGURATION-PROVEN host firewall: nftables INPUT policy DROP; no UDP/9995 accept rule |
| .202 second local nfcapd | UDP/2055 via nfdump@default.service → /var/cache/nfdump (~88M historical); not current Cisco destination |
| .222:2055 | Owner confirmed PRTG; further inspection intentionally excluded as unrelated to DTI primary path |
| DTI role | DIAGNOSTIC / cross-check only |

Post-review option: If .202:9995 NetFlow is retained as a DTI diagnostic input, fix its firewall admission separately. Do not fix merely because the defect exists; first confirm the path is still required.

# 13. CaptureScopeBinding / Site Mapping

| Binding element | Proven value |
| --- | --- |
| capture_source_id | zefer-span-01 |
| sensor site_id | 6a64f17630da7c70d232187a |
| guest CIDR | 192.168.8.0/22 |
| Same Site ID in CaptivPortal | CAPPORT config, Fingerprint sensor, Fingerprint portal, traffic/home modules |
| Production telemetry | Same site_id used for AP/client observation cycles; AP-1/AP-2 EAP613 observed |
| Physical scope | VLAN20 / guest subnet / RSPAN path proven independently |

Verdict: CAPTURE_SCOPE_TO_SITE_PROVEN=YES for current production scope. The binding is not based on a single env variable; it is corroborated across runtime configuration, product modules and production telemetry.

# 14. ResourceBudgetBaseline

## 14.1 Fingerprint + Suricata 60-second service interval

| Service | CPU delta / 60 s | Approx one-core utilization | MemoryCurrent | Tasks |
| --- | --- | --- | --- | --- |
| fingerprint-sensor | 0.502 s | ~0.84% | ~45.1 MiB | 5 |
| fingerprint-suricata | 0.458 s | ~0.76% | ~28.6–28.8 MiB | 6 |
| Combined | ~0.960 s | ~1.60% of one core (~0.40% of total 4-vCPU capacity) | ~74 MiB | 11 |

## 14.2 VM headroom

| Resource | Observed |
| --- | --- |
| vCPU | 4 |
| Load average | 3.60 / 3.46 / 3.10 at snapshot; not equivalent to CPU saturation |
| RAM | 7.7 GiB total; ~1.8 GiB used; ~5.6 GiB available |
| Swap | 4.0 GiB total; ~811 MiB allocated; vmstat si=0/so=0 during sample |
| Root filesystem | 98G total; 54G used; ~40G available; 58% used |
| vmstat interval | ~16.5% user, 11.0% system, 61.4% idle, 10.9% I/O wait; avg runnable ~1.18; avg blocked ~0.64 |
| Interpretation | CPU saturation not observed; active swap pressure not observed; moderate I/O wait is present |

# 15. Storage Baseline

| Storage item | Measured result |
| --- | --- |
| Fingerprint sensor directory | ~90 MiB |
| spool.sqlite3 physical size | 93,708,288 bytes (~89.37 MiB) |
| SQLite page_size / page_count | 4096 / 22,878 |
| SQLite freelist_count | 22,872 pages |
| Approx pages not on freelist | 6 pages ≈ 24 KiB |
| Current spool rows from telemetry | 0–6 in observed samples |
| CaptivPortal logs | ~1.3 GiB |
| Log composition | Mostly rotated application logs around 50 MiB each: auth_telemetry, pending_session_cleaner, omada_webhook* |
| Current disk pressure | NOT OBSERVED |
| Future DTI flow/DNS storage | MODELED ONLY — outputs are not currently enabled, so no honest measured growth baseline exists |

# 16. Natural Traffic / Protocol Sample

A bounded 30 s passive sample on enp8s0 observed 672,803 IPv4-formatted lines, 130 IPv6-formatted lines, 262 ICMP and 51 ARP. The attempted awk TCP/UDP classification did not match tcpdump -q formatting, therefore TCP/UDP counts from that command are UNKNOWN rather than zero. Suricata production stats expose capture health but not protocol-mix counters in the current Fingerprint telemetry. The block is therefore PARTIAL and sufficient for DTI-00; no further visitor-traffic probing was justified.

# 17. ControlledClientTraceProof — Owner Decision

Owner-deferred: The controlled owner-phone trace was intentionally not executed at this stage because existing passive/configuration evidence already proves guest MAC/private IPv4 visibility, both directions and pre-NAT placement. It will be reopened only if a later architecture or QA gate requires identity-correlated evidence that cannot be obtained from existing evidence.

Formal verdict: CONTROLLED_CLIENT_TRACE=PARTIAL. This is a deliberate research-scope decision, not a technical failure.

# 18. Fingerprint / Suricata Coexistence Decision

The accepted architecture defines three possible coexistence topologies: A) reuse existing Suricata with an additional versioned DTI output proven not to affect Fingerprint, B) separate DTI Suricata instance, C) another proved safe fan-out. Current evidence strongly favors A operationally because the existing Suricata already captures the mirror, supports required metadata families and has low current resource cost. However A is not formally “proven not to affect Fingerprint” because DTI-00 correctly did not enable new production outputs.

| Option | Assessment |
| --- | --- |
| A — reuse existing Suricata | PREFERRED CANDIDATE. Lowest operational duplication; capabilities present; current resource cost low. Admission still requires controlled compatibility test of additional versioned output. |
| B — separate DTI Suricata | Not justified by current evidence; would add another full-traffic parser and duplicate capture/CPU/memory/operational burden. |
| C — other fan-out | No concrete safer/better arrangement proven during DTI-00. |

FINGERPRINT_SURICATA_COEXISTENCE_RECOMMENDATION = UNRESOLVED
PREFERRED_NEXT_CANDIDATE = A

# 19. Traffic Accounting Source Decision

Current evidence is sufficient to reject the Fingerprint raw parser as a general traffic meter: its BPF is intentionally limited to DHCP and guest outbound TCP SYN, and its semantics are frozen. Suricata supports flow events and directional packet/byte counters, but DTI-00 did not enable flow output and did not prove the periodic/current-rate/visit-boundary behavior required by the accepted DTI architecture.

| Candidate | DTI-00 result |
| --- | --- |
| Fingerprint raw | NOT SUITABLE AS-IS for general accounting; intentionally filtered and specialized. |
| Suricata flow | STRONG CANDIDATE for flow lifecycle/metadata; terminal-flow semantics may be insufficient for periodic rate/current activity/accurate Visit slicing/long-lived accounting. |
| Lightweight traffic meter | NOT YET PROVEN NECESSARY, but remains an explicitly valid candidate for bounded periodic normalized traffic slices if Suricata terminal flow events are insufficient. |
| NetFlow | DIAGNOSTIC / aggregate cross-check only; not authoritative device identity/accounting. |
| Zeek | NO baseline requirement; no concrete gap found. |

TRAFFIC_ACCOUNTING_SOURCE_RECOMMENDATION = UNRESOLVED
PREFERRED RESEARCH DIRECTION = Suricata for general metadata + validate whether a lightweight periodic meter is needed for accounting semantics

# 20. Zeek Gap Analysis

No concrete capability gap was identified that requires a second heavy full-traffic analyzer. Existing custom Fingerprint + Suricata + a possible lightweight accounting collector cover the current architecture needs under investigation. “Zeek can do more” is not a valid admission reason.

ZEEK_REQUIRED = NO

# 21. Parser Allocation Recommendation

| Fact family | Recommended producer | Evidence / rationale |
| --- | --- | --- |
| Fingerprint DHCP | Existing Fingerprint raw | Frozen BPF + DHCP parser already accepted in production |
| Fingerprint TCP SYN | Existing Fingerprint raw | Frozen guest outbound SYN parser already accepted |
| TLS fingerprint / JA4 | Existing Suricata → Fingerprint | Production-enabled and normalized |
| QUIC fingerprint / JA4 | Existing Suricata → Fingerprint | Production-enabled and normalized |
| Capture health | Suricata stats + NIC counters | Kernel/parser/NIC evidence already available |
| DNS metadata | Suricata (future versioned DTI output) | Installed version supports DNS; not currently enabled in Fingerprint config |
| TLS/QUIC general metadata | Suricata | Natural general metadata role; do not duplicate parser |
| Flow lifecycle | Suricata candidate | Installed flow capability; output not currently enabled |
| Periodic bytes/rate/current activity | UNRESOLVED: validate Suricata vs lightweight meter | Architecture requires periodic/Visit-aware semantics; terminal flow events may be insufficient |
| Security alerts | Suricata only if future DTI security scope admits them | Not part of current accounting baseline |
| NetFlow | Diagnostic cross-check | Different observation/identity semantics |
| Zeek | None | No justified gap |

# 22. Resource and Evidence Degradation Policy Recommendation

| Condition | Required behavior |
| --- | --- |
| Capture drops/misses rise | Mark CaptureQuality degraded/partial; do not present traffic accounting as complete/authoritative. |
| Suricata/EVE unavailable or truncated | Fingerprint raw DHCP/SYN remains independent; DTI Suricata-derived facts become unavailable/stale; never convert missing to zero. |
| CPU/RAM/I/O pressure exceeds admitted budget | DTI loses/degrades its own observations first. Portal/Auth/CAPPORT/Omada/Fingerprint are protected. |
| Queue/storage pressure | Bounded drop/reject + telemetry; no unbounded queue growth; no persistent full packet archive. |
| Mirror duplication present | Traffic byte/packet accounting is non-authoritative until remediation/duplicate semantics are frozen. |
| Site attribution ambiguous | Do not guess site_id; do not enter Site-specific per-device results. |
| Source outage | Historical retained summaries may remain readable with provenance; current DTI status becomes stale/unavailable/insufficient. |

# 23. DTICaptureCapabilityMatrix

| Capability | Status | Evidence | Limitation / note |
| --- | --- | --- | --- |
| client MAC | AVAILABLE | E12/E14 | Visible pre-NAT at capture point. |
| client IPv4 | AVAILABLE | E12/E14/E17 | Guest private IPv4 visible. |
| client IPv6 | PARTIAL | E35 | IPv6 observed, not identity-correlated to owner client. |
| VLAN identity | PARTIAL | E02/E03/E12/E15 | Source VLAN20 scope proven; original VLAN tag not preserved at VM API. |
| Site attribution | AVAILABLE | E32 | zefer-span-01 → Site 6a64… → guest CIDR proven. |
| upload direction | AVAILABLE | E14 | Client→remote observed. |
| download direction | AVAILABLE | E14 | Remote→client observed. |
| pre-NAT identity | AVAILABLE | E12/E17/E18 | Capture before Cisco NAT. |
| both flow directions | AVAILABLE | E14 | Bidirectional visibility observed. |
| bytes | PARTIAL | E42–E48 | Systematic RSPAN duplication remediated; raw byte visibility exists, but authoritative DTI byte basis and periodic accounting semantics remain unfrozen. Low-rate residual exact repeats remain observable. |
| packets | PARTIAL | E42–E48 | Systematic ~50% mirror duplication eliminated; packets are observable, but residual low-rate exact repeats and final accounting semantics still require explicit treatment. |
| flow count | PARTIAL | E30 | Suricata supports flow; production output not enabled; periodic/current semantics remain unproven. Original systematic RSPAN duplication is remediated. |
| TCP visibility | AVAILABLE | E12/E28 | Observed; Fingerprint raw has SYN subset. |
| UDP visibility | AVAILABLE | E12/E21 | Observed including NetFlow delivery. |
| ARP | AVAILABLE | E35 | Observed in bounded sample. |
| DHCP | AVAILABLE | E28 | Fingerprint raw filter/parser explicitly covers DHCP. |
| DNS | PARTIAL | E30 | Suricata supports DNS; not currently enabled in Fingerprint EVE. |
| TLS metadata | AVAILABLE | E26/E27 | Production TLS/JA4 path active. |
| QUIC metadata | AVAILABLE | E26/E27 | Production QUIC/JA4 path active. |
| ICMP | AVAILABLE | E35 | Observed in bounded sample. |
| mDNS | UNKNOWN | — | Not specifically validated; no extra probing justified. |
| SSDP | UNKNOWN | — | Not specifically validated; no extra probing justified. |
| capture drop evidence | AVAILABLE | E23/E24/E29/E47 | NIC + Suricata counters available; post-fix Suricata drop counter stayed 4,098 while packet counter increased. |
| mirror oversubscription evidence | PARTIAL | E24 | Baseline rate/headroom measured; current RX-only remediation reduces avoidable duplicate load, but synchronized post-fix source/destination reconciliation is not complete. |
| parser drop evidence | AVAILABLE | E29 | Suricata kernel drops and EVE health telemetry. |
| event timestamps | AVAILABLE | E27/E29 | Kernel timestamps + EVE timestamps/telemetry. |
| host capture path | AVAILABLE | E04–E11 | End-to-end host path proven. |
| controlled client correlation | PARTIAL | E36 | Owner-deferred as redundant; reopen if later required. |
| NetFlow cross-check | PARTIAL | E19–E21 | Exporter/delivery proven; .202 collector blocked by firewall; .222 PRTG excluded from deeper DTI scope. |

# 24. Unknowns, Limitations and Non-Claims

The original mass RSPAN duplication root cause is no longer an open unknown. Operationally, the systematic defect was tied to VLAN-source direction BOTH in this topology: changing only BOTH → RX removed new Fingerprint exact duplicates while retaining both VLAN20 directions. However, low-rate exact-frame repeats remained in broad physical post-fix captures (~0.28–0.66%); their exact origin was not separately classified and they remain part of future traffic-accounting/dedup semantics.

Absolute mirror completeness is not proven. Current bounded quality evidence is good; oversubscription remains POSSIBLE.

Exact byte semantics for DTI (L2 vs IP vs transport etc.) are not frozen by this task.

Periodic accounting semantics required for current rate/Visit slicing/long-lived flows remain unresolved; Suricata terminal flow capability alone is not sufficient proof.

IPv6 was observed but no controlled IPv6 client correlation was performed.

mDNS/SSDP exact coverage was not separately validated.

Upstream public-NAT placement after Cisco3845 was not required and is not claimed.

The historical 52.29% Fingerprint duplicate ratio applies only to the frozen raw BPF subset and to lifetime counters containing the pre-fix period; it does not describe current post-remediation traffic. Post-fix validation observed +1,345 raw candidates and +0 new Fingerprint exact duplicates over the 14:19–14:30 window.

# 25. Changes Made During Research and Post-Research Remediation

| Change | Authorization / impact |
| --- | --- |
| Install tcpdump on omv-hp | Explicit separate Owner authorization for bounded DTI-00 diagnostic evidence. tcpdump 4.99.3 / libpcap 1.10.3 observed after install. |
| SPAN/RSPAN configuration | POST-RESEARCH OWNER-AUTHORIZED REMEDIATION: source VLAN10,20 direction changed from implicit BOTH to RX-only. Destination RSPAN VLAN999 unchanged. Change validated before persistence. |
| Switch VLAN/trunk configuration | No VLAN membership, trunk allowed-list, native VLAN, or physical port configuration changed. Only monitor-session source direction changed. |
| Cisco routing/NAT/ACL | NONE |
| Omada configuration | NONE |
| Fingerprint service/config | NONE |
| Suricata production config/output | NONE |
| Firewall policy | NONE |
| NIC offload/ring tuning | NONE |
| Startup-config persistence | After validation, running-config was saved. startup-config explicitly contains: monitor session 1 source vlan 10 , 20 rx / monitor session 1 destination remote vlan 999. |

# 26. Post-DTI-00 RSPAN Remediation Record

Owner-required remediation is COMPLETE. The report preserves the original baseline finding and the corrective chronology rather than rewriting history: baseline source VLAN10+VLAN20 used direction BOTH and produced systematic duplication; post-research source direction RX-only eliminated that mass duplicate pattern while preserving VLAN20 bidirectional visibility and capture-service health.

1. Pre-change confirmation: show monitor session 1 reported Source VLANs Both: 10,20; running-config contained monitor session 1 source vlan 10 , 20 (implicit BOTH); Gi1/0/48 continued to carry RSPAN VLAN999.

2. Corrective change: remove the implicit-BOTH source line and configure monitor session 1 source vlan 10 , 20 rx. RSPAN destination VLAN999, VLAN membership and trunk topology were left unchanged.

3. Immediate switch validation: show monitor session 1 reported RX Only: 10,20; running-config explicitly showed monitor session 1 source vlan 10 , 20 rx.

4. Fingerprint duplicate validation: duplicate_packet_count stopped increasing. From 14:19 to 14:30 AZST raw_packet_count increased by 1,345 while duplicate_packet_count increased by 0.

5. VLAN20 direction validation at VM capture NIC: upload and download were both present (41 captured client→network; 383 captured network→client), with 0 kernel drops in both bounded tests.

6. Physical RSPAN validation at omv-hp: 3,599-packet sample contained 10 exact repeats within 1 ms (0.2779%); second 302-packet detailed sample contained 2 (0.6623%). This is orders of magnitude below the pre-fix Fingerprint duplicate pattern and is recorded as low-rate residual exact repeats, not recurrence of the systematic BOTH-direction defect.

7. Fingerprint/Suricata health validation: both services active; runtime_state=ready; Suricata process active; EVE truncated/invalid counters 0; capture_kernel_drops remained 4,098 while capture_kernel_packets continued to increase.

8. Persistence: after all validation gates passed, the switch configuration was saved. startup-config was re-read and confirmed monitor session 1 source vlan 10 , 20 rx and destination remote vlan 999.

Remaining optional issue: if 192.168.0.202:9995 NetFlow remains required, admit UDP/9995 through host firewall under its own reviewed change and revalidate nfcapd ingest. This is independent of the completed RSPAN remediation.

Remaining optional tuning candidate: evaluate RX ring increase from 256 toward supported 4096 only after review/benchmark. Historical misses alone are not authorization to tune, and the RSPAN remediation did not modify NIC ring/offload settings.

# 27. Final DTI-00 Verdict

| Field | Value | Note |
| --- | --- | --- |
| TASK | TASK-DTI-00 |  |
| MODE | RESEARCH / LAB / INVENTORY |  |
| CAPTURE_TOPOLOGY_PROVEN | YES |  |
| CAPTURE_PLACEMENT_PROVEN | YES |  |
| HOST_CAPTURE_PATH_PROVEN | YES |  |
| CONTROLLED_CLIENT_TRACE | PARTIAL | Owner-deferred as redundant at current stage |
| CLIENT_MAC_VISIBILITY | YES |  |
| VLAN_VISIBILITY | PARTIAL | Source VLAN scope proven; original VLAN tag not retained at VM capture presentation |
| PRE_NAT_VISIBILITY | YES |  |
| BOTH_FLOW_DIRECTIONS | YES |  |
| CAPTURE_PRESENTATION_EFFECTS | ACCOUNTED |  |
| DUPLICATE_RISK | POSSIBLE | Original systematic BOTH-direction RSPAN duplication is REMEDIATED/VALIDATED. Low-rate residual exact-frame repeats (~0.28–0.66% in two physical samples) remain observable and must be explicitly handled by future accounting semantics. |
| MIRROR_OVERSUBSCRIPTION_RISK | POSSIBLE |  |
| CAPTURE_COMPLETENESS_EVIDENCE | PARTIAL |  |
| CAPTURE_SCOPE_TO_SITE_PROVEN | YES |  |
| CAPTURE_QUALITY_BASELINE | PASS |  |
| RESOURCE_BASELINE | PASS |  |
| FINGERPRINT_SURICATA_COEXISTENCE_RECOMMENDATION | UNRESOLVED | Option A is preferred candidate but additional DTI output was not admitted/tested under DTI-00 |
| TRAFFIC_ACCOUNTING_SOURCE_RECOMMENDATION | UNRESOLVED | Suricata is preferred metadata/flow candidate; lightweight periodic meter remains candidate for rate/Visit semantics |
| ZEEK_REQUIRED | NO |  |
| NETFLOW_ROLE | DIAGNOSTIC |  |
| RESEARCH_PRIVACY_BOUNDARY | PASS |  |
| OPEN_P0 | 0 |  |
| OPEN_P1 | 0 | Original RSPAN systematic-duplication P1 closed by RX-only remediation and post-fix validation. |
| BLOCKERS | 2 items | 1) Formal full PASS lacks ControlledClientTrace by explicit Owner decision. 2) Coexistence/accounting admission still requires separate approved validation. RSPAN duplication is no longer a blocker. |
| CHANGES_MADE | tcpdump diagnostic install + RSPAN BOTH→RX remediation | Both explicitly Owner-authorized. No VLAN/trunk/routing/NAT/Fingerprint/Suricata/firewall configuration changed. |
| DTI_00_VERDICT | PARTIAL | Final evidence-grade verdict. Physical/host baseline proven; systematic RSPAN duplication found and remediated; remaining partials are controlled-trace and architecture-admission decisions. |

Interpretation: PARTIAL remains the correct evidence-grade DTI-00 verdict, but the reason has changed materially. The physical/host capture baseline is substantially proven and the production systematic RSPAN duplication defect discovered during research has now been remediated and validated. PARTIAL now prevents the report from falsely certifying the intentionally deferred owner-client trace or untested Suricata/periodic-accounting admission topology; it no longer reflects an open RSPAN duplication P1.

# Appendix A — Canonical Evidence Index

| ID | Host / source | Command / observation | Supports conclusion | Class |
| --- | --- | --- | --- | --- |
| E01 | Cisco2960 | show version / clock | Catalyst identity: WS-C2960S-48LPS-L, IOS 15.2(2)E9, serial FOC1846Z2UF. | OBSERVED |
| E02 | Cisco2960 | show monitor session all; show run ¦ section ^monitor | RSPAN session 1; source VLAN10,20 BOTH; remote destination VLAN999. | CONFIGURATION-PROVEN |
| E03 | Cisco2960 | show vlan id 999; show interfaces trunk | VLAN999 RSPAN enabled; transported on Gi1/0/48 trunk, native40, allowed 40,999. | CONFIGURATION-PROVEN |
| E04 | Cisco2960 + omv-hp | MAC table + ip link | Gi1/0/48 learned 44:1e:a1:61:e8:d9; exact match enp7s0f1. | OBSERVED |
| E05 | omv-hp | ip -d link | enp7s0f1.999 VLAN999; macvtap0 passthru. | CONFIGURATION-PROVEN |
| E06 | omv-hp/libvirt | virsh domiflist | Ubunt VM direct macvtap0 / enp7s0f1.999 MAC 52:54:00:59:23:b8. | CONFIGURATION-PROVEN |
| E07 | Ubuntu VM | ip/link/address observation | enp1s0=192.168.0.202; enp8s0 capture NIC no IP; MAC matches macvtap. | OBSERVED |
| E08 | Ubuntu VM | systemctl/process inventory | Fingerprint classification/evidence/sensor/Suricata services running. | OBSERVED |
| E09 | Ubuntu VM | systemctl show/cat fingerprint-suricata | Suricata 8.0.6 AF_PACKET enp8s0 workers; EVE unix socket; resource limits observed. | CONFIGURATION-PROVEN |
| E10 | Ubuntu VM | sensor.env + ss -xap | capture_source=zefer-span-01; site/cidr configured; EVE unix socket held by sensor. | CONFIGURATION-PROVEN |
| E11 | Ubuntu VM | /proc/net/packet + FD mapping | Two direct packet consumers: fingerprint-sensor + Suricata. | OBSERVED |
| E12 | Ubuntu VM | bounded tcpdump enp8s0 | Guest MAC/private IPv4 and pre-NAT traffic visible; VM capture API untagged. | OBSERVED |
| E13 | Ubuntu VM | bounded packet identity comparison | Exact duplicated packets observed at enp8s0. | OBSERVED |
| E14 | Ubuntu VM | guest-filtered tcpdump | Both outbound and return guest traffic observed. | OBSERVED |
| E15 | omv-hp | tcpdump enp7s0f1 vlan999 and enp7s0f1.999 | Duplicates already present on physical RSPAN ingress; VLAN999 tag visible physically. | OBSERVED |
| E16 | Cisco3845 | show version / interfaces | Cisco3845 identity and interface inventory. | OBSERVED |
| E17 | Cisco3845 | show run/int Gi0/1.20 | Guest first L3 192.168.10.1/22, dot1Q20, NAT inside. | CONFIGURATION-PROVEN |
| E18 | Cisco3845 | Gi0/0 + show ip nat statistics/config | NAT outside on Gi0/0 192.168.100.10; overload ACL100. Capture pre-Cisco-NAT. | CONFIGURATION-PROVEN |
| E19 | Cisco3845 | show ip flow export / config | NetFlow v9 destinations .222:2055 and .202:9995; differs from earlier .220 expectation. | CONFIGURATION-PROVEN |
| E20 | Ubuntu VM | ss/systemctl/cat nfcapd/nfdump | nfcapd :9995 → /var/nfdump; second nfcapd :2055 → /var/cache/nfdump; historical storage measured. | CONFIGURATION-PROVEN |
| E21 | Ubuntu VM | nfdump/journal + tcpdump + nftables | UDP/9995 reaches enp1s0; nfcapd sees 0 flows; nftables INPUT DROP has no UDP/9995 allow. | CONFIGURATION-PROVEN |
| E22 | omv-hp | ethtool -i/-k + lspci | Intel 82576 / HP NC362i, igb, firmware 1.8.2; 1Gb/full; offloads inventoried. | OBSERVED |
| E23 | omv-hp | ethtool -S | Historical rx_missed/fifo=46121, no_buffer=2074, CRC=1; queue drops zero. | OBSERVED |
| E24 | omv-hp | 60 s before/after counters | ~255.9 Mb/s physical, ~156.6 Mb/s VLAN999; no new miss/fifo/no-buffer/CRC/VLAN999 drop. | OBSERVED |
| E25 | omv-hp | ethtool -g | RX ring current 256, max 4096; TX same. | CONFIGURATION-PROVEN |
| E26 | Ubuntu VM | production suricata.yaml | Current EVE: TLS custom JA4/ALPN/handshake, QUIC, stats only. | CONFIGURATION-PROVEN |
| E27 | Ubuntu VM | Fingerprint normalizer/runtime source inspection | TLS/QUIC/stats EVE are consumed and normalized; Suricata heartbeat/health semantics present. | CONFIGURATION-PROVEN |
| E28 | Ubuntu VM | capture_filter.py + capture.py + dedup.py | Raw BPF = DHCP + guest outbound SYN; exact BLAKE2s dedup 0.25 ms / 65536 before normalizer. | CONFIGURATION-PROVEN |
| E29 | Ubuntu VM | journal fingerprint metrics/stats | raw_dedup_ratio ~52.29% filtered subset; Suricata capture_kernel_drops=4098 stable during sample; runtime ready. | OBSERVED |
| E30 | Ubuntu VM | suricata --build-info + stock /etc/suricata/suricata.yaml | Installed Suricata supports AF_PACKET, flow, DNS, DHCP, HTTP, TLS, QUIC, stats, alert; not all enabled in production. | CONFIGURATION-PROVEN |
| E31 | Ubuntu VM | 60 s systemctl CPUUsageNSec/MemoryCurrent | Sensor ~0.502 CPU-s; Suricata ~0.458 CPU-s; combined ~74 MiB memory. | OBSERVED |
| E32 | Ubuntu VM | site/capture grep across runtime/docs/logs | zefer-span-01 + site 6a64… + 192.168.8.0/22 consistently bound; same Site in production telemetry. | CONFIGURATION-PROVEN |
| E33 | Ubuntu VM | nproc/uptime/free/df + vmstat 5 12 | 4 vCPU; RAM/disk headroom good; no active swap; ~61% avg idle, ~11% I/O wait. | OBSERVED |
| E34 | Ubuntu VM | du + SQLite read-only PRAGMA + log file inventory | Spool physical ~89.4 MiB but 22,872/22,878 pages free; logs 1.3G mostly rotated app logs. | OBSERVED |
| E35 | Ubuntu VM | 30 s bounded tcpdump protocol sample | IPv4 dominant; IPv6/ICMP/ARP observed; TCP/UDP awk classification inconclusive. | OBSERVED / PARTIAL |
| E36 | Owner decision | Explicit scope decision | Controlled owner-client trace deferred as redundant; reopen only if future gate requires identity-correlation. | OWNER-DEFERRED |
| E37 | omv-hp | apt install tcpdump under explicit authorization | tcpdump installed solely for bounded DTI evidence; separate Owner-approved diagnostic change. | CHANGE RECORDED |
| E38 | Owner scope decision | Owner confirmation | 192.168.0.222:2055 is PRTG; deeper collector research excluded as unrelated to primary DTI path. | OWNER-CONFIRMED |
| E39 | Cross-evidence synthesis | E02/E15 + router-on-a-stick topology | Likely duplicate mechanism is multi-VLAN/BOTH mirroring around routed traffic; exact root cause pending post-report corrective test. | STRONGLY SUPPORTED / NOT FINAL |
| E40 | Architecture synthesis | Fingerprint + Suricata + accepted DTI architecture | Fingerprint remains specialized; Suricata general metadata role; periodic accounting source still requires admission proof. | RECOMMENDATION |
| E41 | Owner instruction | Explicit post-DTI requirement | Owner mandated post-DTI duplication remediation; requirement subsequently completed and validated in E42–E48. | MANDATORY FOLLOW-UP / COMPLETED |
| E42 | Cisco2960 | show monitor session 1; show running-config ¦ section ^monitor; show interfaces trunk | Pre-remediation state reconfirmed: source VLAN10,20 BOTH (implicit in running-config); remote destination VLAN999; transport trunk unchanged. | POST-RESEARCH CONFIGURATION-PROVEN |
| E43 | Cisco2960 | no monitor session 1 source vlan 10 , 20; monitor session 1 source vlan 10 , 20 rx; show monitor session 1 | One-variable remediation applied: source VLANs unchanged; direction changed BOTH → RX Only; destination VLAN999 unchanged. | OWNER-AUTHORIZED REMEDIATION / CONFIGURATION-PROVEN |
| E44 | Ubuntu VM | fingerprint_sensor_metrics before/after RX-only | 14:19→14:30 AZST: raw_packet_count 153,229→154,574 (+1,345); duplicate_packet_count stayed 80,087 (+0). Systematic exact-duplicate growth stopped. | POST-REMEDIATION OBSERVED |
| E45 | Ubuntu VM | bounded tcpdump enp8s0 src/dst 192.168.8.0/22 | Bidirectional VLAN20 visibility preserved after fix: upload 41 captured / download 383 captured; 0 kernel drops in both tests. | POST-REMEDIATION OBSERVED |
| E46 | omv-hp | bounded physical VLAN999 PCAP + exact-frame BLAKE2s comparison | Post-fix physical samples: 10/3,599 exact repeats within 1 ms (0.2779%) and 2/302 (0.6623%); 0 tcpdump kernel drops. Mass ~50% pattern no longer present. | POST-REMEDIATION OBSERVED |
| E47 | Ubuntu VM | systemctl is-active + latest fingerprint_sensor_metrics / fingerprint_suricata_stats | fingerprint-sensor and fingerprint-suricata active; runtime ready; EVE invalid/truncated=0; capture_kernel_drops fixed at 4,098 while packet counter increased. | POST-REMEDIATION OBSERVED |
| E48 | Cisco2960 | copy running-config startup-config; show startup-config ¦ section ^monitor | Final remediation persisted: startup-config contains monitor session 1 source vlan 10 , 20 rx and destination remote vlan 999. | POST-REMEDIATION CONFIGURATION-PROVEN |

# Appendix B — Key Command / Observation Inventory

| Host | Primary read-only evidence commands |
| --- | --- |
| Cisco2960 | show monitor session all; show run ¦ section ^monitor; show vlan id 999; show interfaces trunk; MAC table; interface status/counters |
| Cisco3845 | show version; show interfaces; show run interface Gi0/1.20; Gi0/0; show ip nat statistics; show ip flow export; show run ¦ include ip flow-export |
| omv-hp | ip -d link; virsh domiflist; tcpdump bounded captures; ethtool -i/-k/-S/-g enp7s0f1; lspci -s 07:00.1 -nnk; 60 s counter delta |
| Ubuntu VM | ip/link/address; systemctl show/cat; ss; /proc/net/packet; process FD mapping; tcpdump bounded captures; nftables/iptables; nfdump/nfcapd; source inspection under /opt/CaptivePortal/app/device_fingerprint_sensor; Suricata build info/config; systemctl resource deltas; vmstat; SQLite read-only PRAGMAs; storage/log inventory |

# Appendix C — Questions for Architect / Tech Lead Review

1. Accept closure of the original RSPAN systematic-duplication P1: baseline BOTH-direction defect was remediated to RX-only, bidirectional VLAN20 visibility preserved, services healthy, no post-fix drop growth observed, and startup-config persistence verified.

2. Confirm whether Option A (reuse existing Suricata with an isolated/versioned DTI output) should be the first coexistence admission test, or whether isolation requirements force B/C.

3. For periodic traffic accounting, define the exact evidence needed to decide Suricata flow vs lightweight meter: cadence, long-lived flow treatment, Visit slicing, byte basis, retransmission/dedup semantics, and treatment of low-rate residual exact repeats.

4. Confirm whether .202:9995 NetFlow is still required. If yes, authorize a separate firewall correction and revalidation; if no, retire/ignore it rather than repairing unused infrastructure.

5. Decide whether the historical NIC misses justify a controlled RX-ring benchmark now that the systematic RSPAN duplicate load has been removed; no NIC tuning was performed.

6. Accept CONTROLLED_CLIENT_TRACE=PARTIAL/Owner-deferred for this report, or explicitly request one targeted trace if formal PASS is required before DTI-01.

END OF REPORT